East West University

Department of CSE

| Course Code and Name: Electrical Circuit (CSE209) |  |  |
| --- | --- | --- |
| Experiment no: 06 |  |  |
| Experiment name: Verification of Thevenin’s theorem |  |  |
| Semester and Year: Fall 2021 |  |  |
| Name of Student: D.M. Rafiun-Bin-Masud Student Id: 2019-3-60-137 | Course Instructor information: M Saddam Hossain Khan Senior Lecturer, Department of Computer Science and Engineering |  |
| Date of Report Submitted: 29 December,2021 | Pre-Lab Marks: |  |
|  | Post Lab Marks: |  |
|  | TOTAL Marks: |  |

Circuit diagram:

Figure-1

Figure-2

In figure 2,

Voc = 7.090 V

Figure-3

In figure 3,

Isc = 6.034 mA

Rth = Voc/ Isc

= 1.175k

Figure-4

From Fig-1 and Fig-2,

We see this is the Thevenin’s theorem verified.

Experimental Datasheet:

Table 1:

| Measured Value of E1 | Measured Value of E2 | Measured Value of VL | Measured Value of IL | Measured Value of VOC | Measured Value of ISC | Measured Value of resistors (KV) |
| --- | --- | --- | --- | --- | --- | --- |
| 10 V | 5 V | 3.258 mA | 3.3 mA | 7.130 V | 6.10 mA | R1 = 0.985 R2 =3.27 R3 =4.60 R4 =0.989 RL =0.982 |

Comparing the theoretical calculate values with the experimental values:

Table 2:

|  | VL (V) | I(mA) | VOC (V) | ISC (mA) | RTH (V) |
| --- | --- | --- | --- | --- | --- |
| Calculated | 3.24 | 3.30 | 7.08 | 6.10 | 1161 |
| Measured | 3.258 | 3.30 | 7.13 | 6.10 | 1169 |
| Error | 0.5% | 0% | 0.7% | 0% | 0.7% |

From Figure-1 and Figure-2,

We see this is the Thevenin’s theorem verified

Pre-Lab:

POST LAB

For fig 1,

KVL in mesh 1,

4700i1 + 1000(i1-i2) = -10

- 5700i1 - 1000i2 = -10…………….(1)

KVL in mesh 2,

1000(i2 – i1) + 1000i2 + 3300(i2 – i3) =10-5

- -1000i1 + 5300i2 -3300i3 = 5……..(2)

KVL in mesh 3,

3300(i3 – i2) + 1000i3 = 5

- -3300i2 + 4300i3 =5……………..(3)

From (1), (2) and (3)

i1 = -1.274mA

i2 = 2.733mA

i3 = 3.260mA

IL = 3.260mA

VL = IL * RL = (3.260 * 10-3 *1000)

= 3.260V

For fig 2,

KVL in mesh 1,

4700i1 + 1000(i1-i2) = -10

- 5700i1 – 1000i2 = -10……….(1)

KVL in mesh 2,

1000(i2 – i1) + 1000i2 + 3300i2 = 10.5

- -1000i1 + 5300i2 = 5 ………..(2)

From (1) (2),

i1 = -1.643mA

i2 = 0.633mA

VOC = (5 + 0.633*10-3 * 3300)V

= 7.089V

For fig 3,

KVL at mesh 1,

4700i1 + 1000(i1 – i2) = -10

- 5700i1 – 1000i2 = -10…………(1)

KVL at mesh 2,

1000(i2 – i1) + 1000i2 + 3300(i2 – i3) = 10 – 5

- -1000i1 + 5300i2 -3300i3 = 5…..(2)

KVL at mesh 3,

3300(i3 – i2) = 5

- -3300i2 + 3300i3 = 5……………(3)

From (1), (2) and (3)

i1 = -96.153mA

i2 = 4.573mA

i3 = 6.034mA

ISC = 6.034mA

Rth = Voc/Isc = 7.089/6.034*10-3

=1.175kΩ

Eth = 7.089V

For fig 4,

IL = (Eth/ Rth) +  RL

= 7.089/1175 + 1000

= 3.259 mA

VL = RL * IL

= (1000 * 3.259*10-3)

= 3.259V

So, Thevenin’s theorem verified.

2.

| Theoretically calculated values | PSpice Simulation values |
| --- | --- |
| IL = 3.260mA (Fig-1) | IL = 3.260mA (Fig-1) |
| VL = 3.260V (Fig-1) | VL = 3.260V (Fig-1) |
| Voc = 7.089V | Voc = 7.090 V |
| ISC = 6.034mA | ISC = 6.034mA |
| Rth= 1175Ω | Rth= 1175 |
| IL = 3.260mA (Fig-4) | IL = 3.260mA (Fig-4) |
| VL = 3.260V (Fig-4) | VL = 3.260V (Fig-4) |

There is no discrepancy in PSpice simulated values and the calculated values.

3.

By simulating those 4 circuits we got the values of VL, IL, VOC, ISC, Rth, Eth and got the same values by calculation which are same. So, the Thevenin’s Theorem is verified.

CONCLUSIONS

From this experiment we learnt how to use Thevenin’s theorem and also verified the Thevenin’s theorem by the measured data and the calculated data.